TEST SETUP & TAKING WEIGHT TARE MEASUREMENTS

- Test team take their positions.

  - Test Conductor (TC) stands between tunnel operator station and test section.

  - Tunnel Operator (TO) seated at tunnel operation console and computer.

  - Data Recorder (DR) seated to the right of tunnel operator.

  - Model Observers (MO) standing at wind tunnel test section observation window.

- At the tunnel operation computer, set data collection data file:

  - Select “Define Files”

  - For data file, create new folder “ARO101-Section 6”

  - In the newly created folder, set file name to “ARO101_4PM.csv”

  - Click OK

- At tunnel control panel, set/verify:

  - Tunnel power OFF (switch in down position)

  - VFD OFF (light NOT illuminated)

  - Tunnel speed knob = FULL LEFT (zero velocity)

- At tunnel operation computer, set/verify:

  - Run # = 0.

  - Test # = 1.

  - Point # = 1.

- Set/verify wing Angle Of Attack (Pitch) to 0 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 2 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 5 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 8 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 10 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 12 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to 14 DEG.

- Take data.

- Set/verify wing Angle Of Attack (Pitch) to -2 DEG.

- Take data.

- Set  Run# = 1.

- Set Point# =1.

TEST PROCEDURE FOR A SINGLE GROUP’S TEST RUN

- Set/Verify Run# = Your group’s number and Point# = 1

- Set tunnel velocity to ____  feet/second	.	(30, 50, 70, 90, 110, and 130)

- Take data.

- Set wing Angle of Attack to 2 DEG.

- Take data.

- Set wing Angle of Attack to 5 DEG.

- Take data.

- Set wing Angle of Attack to 8 DEG.

- Take data.

- Set wing Angle of Attack to 10 DEG.

- Take data.

- Set wing Angle of Attack to 12 DEG.

- Take data.

- Slowly set tunnel velocity to ZERO feet/second.

- Set wing Angle of Attack to 14 DEG.

- Slowly increase tunnel velocity back to _____ feet/second.

- Take data.

- Set wing Angle of Attack to -2 DEG.

- Take data.

- Set wing Angle of Attack to 0 DEG.

- Increment Run# to next group’s number.

- Set Point# =1.

REPEAT ABOVE FOR EACH TEAM, WHERE EACH TEAM RUNS A NEW VELOCITY.

TUNNEL SHUT DOWN PROCEDURE

- Set/verify wing Angle Of Attack to 0 DEG.

- Slowly set tunnel velocity to ZERO feet/second.

- Set VFD = OFF (light turns off).

- Set tunnel power = OFF (switch in DOWN position).